The Clorox Company

1221 Broadway Oakland, CA 94612 Tel. (510)271-7000

Material Safety Data Sheet

Product:

Description:

Other Designations

PINE-SOL® BRAND CLEANER

CLEAR AMBER LIQUID WITH CHARACTERISTIC PINE ODOR

Emergency Telephone Nos.

Distributor

For Medical Emergencies call:

(800)446-1014

Clorox Sales Company '1221 Broadway Oakland, CA 94612

For Transportation Emergencies Chemtrac
		(800) 424-9300

II Health Hazard Data

111 Hazardous Ingredients

Eye irritant. May cause skin irritation. Avoid contact with food. No medical conditions are known to be aggravated by exposure to this product. FIRST AID:

Pine oil

CAS # 8002-09-3 Isopropyl alcohol

CAS # 67-63-0

EYE CONTACT:   Immediately rinse eyes with water for 15 minutes. If irritation persists, call a physician.

:;INGESTION:   DO NOT INDUCE VOMITING.  Drink a glassful of water. Immediately call a physician.

SKIN CONTACT:   Remove contaminated clothing. Wash skin with water, irritation persists, call a physician.

INHALATION: Remove to fresh air. If breathing problems develop, call a physician.

Worker Exposure Limit Not established.

Ingredient

Concentration 10-20%

4 - 7%

400 ppm - TLV-TWA3 400 ppm - PEL" 500 ppm - TLV-STEL0 Not established.

2 - 6%

Aikyl alcohol ethoxylates

CAS #66455-15-0

aTLV-TWA = ACGIH Threshold Limit Value - Time Weighted Average "PEL-TWA = OS HA Permissible Exposure Limit - Time Weighted Average TLV-STEL = ACGIH Threshold Limit Value'- Short Term Exposure Limit None of the materials in this product are on the IARC, OSHA, or NTP carcinogen lists.

IV Special Protection and Precautions

V Transportation and Regulatory Data

Hygienic Practices: Wash hands after direct contact. Do not wear product-contaminated clothing for prolonged periods.

Engineering Controls: Use local exhaust to minimize exposure to product mist.

Personal.Protective Equipment: Wear safety glasses. Wear gloves if there is the potential for repeated or prolonged skin contact.

DOT: Not restricted.

DOT Proper Shipping Name: n/a

IMP Proper Shipping Name:Dangerous goods in limited quantities of Class

3.3.      >*...

IATA Proper Shipping Name:Flarnmable liquid, Class 3, PGIII.

TSCA Inventory/Canadian Domestic Substances List Status:

All components of this product are listed.

EPA - SARA Title III/CERCLA:  Packaged product is not reportable under

Sections 311/312. This product contains no chemicals regulated under

Section 313 and contains sodium dodecylbenzenesulfonate

(CAS #25155-30-0, <4%), ammonium hydroxide (CAS #1336-21-6, <1%),

benzole acid (CAS #65-85-0, <1%) and glycol ethers (<1%) which are

regulated under Section 304/CERCLA.

VI Spill Procedures/Waste Disposal

VII Reactivity Data

Spill Procedures: Absorb and containerize. Wash residual down to sanitary sewer. Contact the sanitary treatment facility in advance to assure ability to process washed-down material.

Waste Disposal: Dispose of in accordance with all applicable federal, state, and local regulations.

Stable under normal use and storage conditions.

VIII Fire and Explosion Data

IX Physical Data

Flash Point:   105°F (Setaflash closed cup)

Fire Extinguishing Agents:   Dry chemical, carbon dioxide (CO2), foam or

water spray.

pH	3.0-4.0

Specific gravity	-1-0

Solubility in water	Soluble

*~

S1963, 1991 THE CLOROX COMPANY DATA SUPPLIED IS FOR USE ONLY IN CONNECTION WITH OCCUPATIONAL SAFETY AND HEALTH

DATE PREPARED   10/00